Part F - DAILY AND ANNUAL QUANTITIES AND SHRINKAGE

1 General

1.1 For the purposes of this Part F the CSEP shall be treated as comprising separately;

(a) a Connected System Exit Point in respect of all the NDM Supply Points

(“NDM CSEP”);

(b) a Connected System Exit Point in respect of all the DM Supply Points

(“DM CSEP”).

2 Demand Determination – DM Supply Points

2.1 It is acknowledged by Pipeline Users that the Large Transporter will require for the purposes of the UNC that DM Output Nominations be made to it in respect of each DM Supply Point by the relevant CSEP User and that accordingly, the Pipeline Operator shall not have any responsibility to make the same.

2.2 The quantity of gas offtaken at a DM Supply Point in respect of a Day (“DM Demand”) shall be determined as the aggregate of the quantities offtaken at each DM Supply Meter Point comprised in that DM Supply Point as determined by the Large Transporter by Daily Meter Readings or estimation. It is acknowledged that the DM Demand as so determined will be used by the Large Transporter or the purposes of the UNC and in particular for determining the UDQOs of CSEP Users in respect of the DM CSEP.

3 Demand Determination – NDM Supply Points

3.1 The UNC provides (in Section H of the UNC) for the establishment of Demand Models and End User Categories for each Gas Year.

3.2 Pipeline Users acknowledge that it is aware of the consultation process provided for in the UNC in respect of such matters.

3.3 The Pipeline Operator will adopt and apply for each Gas Year the End User Categories (of NDM Supply Point) established by the Large Transporter pursuant to Section H1.9 of the UNC, and will allocate each NDM Supply Meter Point to one of such categories.

4 Determination of NDM Annual Quantities

4.1 The Pipeline Operator will determine the Annual Quantity ("AQ") for a Supply Meter Point for a Gas Year in accordance with Part CI 6.

5 Records

5.1 The Pipeline Operator will maintain records for each NDM Supply Meter Point of the Annual Quantities and values of any other variable ("a relevant variable") by reference to which End User Categories are (in any Gas Year) defined.

5.2 The Pipeline User will provide to the Pipeline Operator any information relating to any

relevant variable to enable the Pipeline Operator to comply with Clause 5.1.

6 Relevant Metered Period

NOT USED.

7 NDM Offtake Quantities

7.1 The quantities nominated for offtake by Pipeline Users each Day from the Large Transporter System at the NDM CSEP, and the quantities determined (after each Day) to have been so offtaken at the NDM CSEP, will be determined on the same basis as is provided for in Sections C, H and E  of the UNC

8 Calculation by the Pipeline Operator in respect of NDM Supply Points

NOT USED.

9 NDM Supply Meter Point Demand

9.1 The Large Transporter will calculate NDM Supply Meter Point Demand in respect of each  Supply Point as per Section C of the UNC.

10 Reconciliation – DM CSEP

10.1 It is acknowledged that reconciliation in respect of the DM CSEP for the purposes of the UNC will be carried out by the Large Transporter pursuant to the terms of the LDZ CSEP Ancillary Agreement.

11 Reconciliation – NDM CSEP

11.1 It is acknowledged that for the purposes of the UNC, the Large Transporter will determine Supply Point Reconciliation Quantities. Reconciliation in respect of Supply Points will be determined by the Large Transporter pursuant to the terms of the UNC.

11.2 Notwithstanding the fact that the Large Transporter may require Supply Point Reconciliation under the UNC, the Pipeline Operator shall not be required to make any adjustment to any Invoice Amount as a result thereof unless the Pipeline Operator agrees that without such adjustment, the Pipeline User would make or would have made a payment which is materially more or less than that which would be or have been required if Supply Point Reconciliation were to be taken into account.

12 Pipeline shrinkage

12.1 For the purposes of this Code:

(a) "Pipeline Shrinkage": means gas offtaken from the Large Transporter System at the CSEP which is lost from or unaccounted for as offtaken from the Pipeline, including gas lost or unaccounted for by reason of leakage, theft, meter error and meter correction.

(b) "Pipeline Shrinkage Factor": is a factor calculated (as an average for all Days in the Gas Year), for any quantity to be offtaken from the Large Transporter System at the CSEP, as the sum of such quantity and the Pipeline Shrinkage attributable thereto divided by such quantity.

13 Calculation of Pipeline Shrinkage Factor

13.1 Not later than 1st August in each Gas Year the Pipeline Operator shall:

(a) in good faith and as accurately as is reasonably practicable:

(i) assess whether the Pipeline Shrinkage Factor will in the current Gas Year have accurately reflected Pipeline Shrinkage; and

(ii) estimate (in accordance with Clause 13.2) the value of the Pipeline Shrinkage Factor to apply in the following and in each Gas Year;

(b) provide to the Large Transporter such estimated value and assessment and details (sufficient to enable the Large Transporter to verify the same) of the basis on which such value was estimated and assessment was made.

13.2 The value of the Pipeline Shrinkage Factor to apply in a particular Gas Year shall be

established so as to take account of the extent by which (in accordance with Clause 13.1(a)(i) it is assessed that Pipeline Shrinkage in the preceding year will be over or under-estimated.

13.3 Each Pipeline User acknowledges that the Pipeline Shrinkage Factor in respect of the

First Gas Year is as set out in the IGTAD.

13.4 Each Pipeline User shall provide to the Pipeline Operator in a timely manner all relevant information in order to help the Pipeline Operator to comply with its obligations under clause 13.1(a).

13.5 The Pipeline Operator shall negotiate in good faith with the Large Transporter in respect of the determination of the Pipeline Shrinkage Factor, and shall use (as far as is reasonably practicable) the information provided to the Pipeline Operator by Pipeline Users in accordance with Clause 13.4.